.model small

.stack 100h

.data

msg db 10, 13, "Enter a string (maximum 255 characters): $"

result db 10, 13, "Number of operators: $"

.code

main proc

mov ax, @data

mov ds, ax

lea dx, msg

mov ah, 9

int 21h ; Display the prompt message

mov si, 0 ; Initialize index for input string

mov cx, 0 ; Initialize operator count

read_loop:

mov ah, 1

int 21h ; Read a character from keyboard

cmp al, 13 ; Check if ENTER key pressed

je done ; If yes, exit the loop

; Check if the character is an operator (+, -, /, *)

cmp al, '+' ; Add more operators as needed

je increment_count

cmp al, '-'

je increment_count

cmp al, '/'

je increment_count

cmp al, '*'

je increment_count

; If not an operator, continue reading

inc si

jmp read_loop

increment_count:

inc cx ; Increment operator count

inc si

jmp read_loop

done:

; Display the operator count

mov ah, 2

lea dx, result

int 21h

mov al, cl ; Load the count into AL

add al, '0' ; Convert to ASCII

mov ah, 0

int 10h ; Display the count

mov ah, 4Ch

int 21h ; Exit program

main endp

end main

.model small

.stack 100h

.data

msg db 10, 13, "Enter a string (maximum 255 characters): $"

result db 10, 13, "Number of uppercase letters: $"

.code

main proc

mov ax, @data

mov ds, ax

lea dx, msg

mov ah, 9

int 21h ; Display the prompt message

mov si, 0 ; Initialize index for input string

mov cx, 0 ; Initialize uppercase letter count

read_loop:

mov ah, 1

int 21h ; Read a character from keyboard

cmp al, 13 ; Check if ENTER key pressed

je done ; If yes, exit the loop

; Check if the character is an uppercase letter

cmp al, 'A'

jb not_uppercase

cmp al, 'Z'

ja not_uppercase

inc cx ; Increment uppercase letter count

not_uppercase:

inc si

jmp read_loop

done:

; Display the uppercase letter count

mov ah, 2

lea dx, result

int 21h

mov al, cl ; Load the count into AL

add al, '0' ; Convert to ASCII

mov ah, 0

int 10h ; Display the count

mov ah, 4Ch

int 21h ; Exit program

main endp

end main

.model small

.stack 100h

.data

msg db 10, 13, "Enter a string (maximum 255 characters): $"

result db 10, 13, "Number of lowercase letters: $"

.code

main proc

mov ax, @data

mov ds, ax

lea dx, msg

mov ah, 9

int 21h ; Display the prompt message

mov si, 0 ; Initialize index for input string

mov cx, 0 ; Initialize lowercase letter count

read_loop:

mov ah, 1

int 21h ; Read a character from keyboard

cmp al, 13 ; Check if ENTER key pressed

je done ; If yes, exit the loop

; Check if the character is a lowercase letter

cmp al, 'a'

jb not_lowercase

cmp al, 'z'

ja not_lowercase

inc cx ; Increment lowercase letter count

not_lowercase:

inc si

jmp read_loop

done:

; Display the lowercase letter count

mov ah, 2

lea dx, result

int 21h

mov al, cl ; Load the count into AL

add al, '0' ; Convert to ASCII

mov ah, 0

int 10h ; Display the count

mov ah, 4Ch

int 21h ; Exit program

main endp

end main

.model small

.stack 100h

.data

msg db 10, 13, "Enter a string (maximum 255 characters): $"

result_lowercase db 10, 13, "Number of lowercase letters: $"

result_uppercase db 10, 13, "Number of uppercase letters: $"

.code

main proc

mov ax, @data

mov ds, ax

lea dx, msg

mov ah, 9

int 21h ; Display the prompt message

mov si, 0 ; Initialize index for input string

mov cx, 0 ; Initialize lowercase letter count

mov bx, 0 ; Initialize uppercase letter count

read_loop:

mov ah, 1

int 21h ; Read a character from keyboard

cmp al, 13 ; Check if ENTER key pressed

je done ; If yes, exit the loop

; Check if the character is an uppercase or lowercase letter

cmp al, 'A'

jb not_letter

cmp al, 'Z'

jbe uppercase_found

cmp al, 'a'

jb not_letter

cmp al, 'z'

jbe lowercase_found

not_letter:

inc si

jmp read_loop

uppercase_found:

inc bx ; Increment uppercase letter count

inc si

jmp read_loop

lowercase_found:

inc cx ; Increment lowercase letter count

inc si

jmp read_loop

done:

; Display the lowercase and uppercase letter counts

mov ah, 2

lea dx, result_lowercase

int 21h

mov al, cl ; Load the lowercase count into AL

add al, '0' ; Convert to ASCII

mov ah, 0

int 10h ; Display the lowercase count

lea dx, result_uppercase

int 21h

mov al, bl ; Load the uppercase count into AL

add al, '0' ; Convert to ASCII

mov ah, 0

int 10h ; Display the uppercase count

mov ah, 4Ch

int 21h ; Exit program

main endp

end main

.model small

.stack 100h

.data

msg1 db 10, 13, "Enter the first number: $"

msg2 db 10, 13, "Enter the second number: $"

msg3 db 10, 13, "Enter the operator (+ or -): $"

result db 10, 13, "Result: $"

.code

main proc

mov ax, @data

mov ds, ax

; Input: First number

lea dx, msg1

mov ah, 9

int 21h

mov ah, 1

int 21h

sub al, '0' ; Convert ASCII to numeric value

mov bl, al ; Store the first number

; Input: Second number

lea dx, msg2

mov ah, 9

int 21h

mov ah, 1

int 21h

sub al, '0' ; Convert ASCII to numeric value

mov bh, al ; Store the second number

; Input: Operator

lea dx, msg3

mov ah, 9

int 21h

mov ah, 1

int 21h

; Calculate result based on the operator

cmp al, '+' ; Check if operator is addition

je add_numbers

cmp al, '-' ; Check if operator is subtraction

je subtract_numbers

; Invalid operator

lea dx, result

mov ah, 9

int 21h

mov dl, '?'

int 21h

jmp done

add_numbers:

add bl, bh ; Add the numbers

jmp display_result

subtract_numbers:

sub bl, bh ; Subtract the numbers

jmp display_result

display_result:

; Display the result

lea dx, result

mov ah, 9

int 21h

mov dl, bl ; Convert result to ASCII

add dl, '0'

int 21h

done:

mov ah, 4Ch

int 21h ; Exit program

main endp

end main

.model small

.stack 100h

.data

msg1 db 10, 13, "Enter the first number: $"

msg2 db 10, 13, "Enter the second number: $"

msg3 db 10, 13, "Enter the operator (+ or -): $"

repeat_msg db 10, 13, "Do you want to repeat? (Y/N): $"

result db 10, 13, "Result: $"

.code

main proc

mov ax, @data

mov ds, ax

repeat_loop:

; Input: First number

lea dx, msg1

mov ah, 9

int 21h

mov ah, 1

int 21h

sub al, '0' ; Convert ASCII to numeric value

mov bl, al ; Store the first number

; Input: Second number

lea dx, msg2

mov ah, 9

int 21h

mov ah, 1

int 21h

sub al, '0' ; Convert ASCII to numeric value

mov bh, al ; Store the second number

; Input: Operator

lea dx, msg3

mov ah, 9

int 21h

mov ah, 1

int 21h

; Calculate result based on the operator

cmp al, '+' ; Check if operator is addition

je add_numbers

cmp al, '-' ; Check if operator is subtraction

je subtract_numbers

; Invalid operator

lea dx, result

mov ah, 9

int 21h

mov dl, '?'

int 21h

jmp repeat_prompt

add_numbers:

add bl, bh ; Add the numbers

jmp display_result

subtract_numbers:

sub bl, bh ; Subtract the numbers

jmp display_result

display_result:

; Display the result

lea dx, result

mov ah, 9

int 21h

mov dl, bl ; Convert result to ASCII

add dl, '0'

int 21h

repeat_prompt:

; Ask if the user wants to repeat

lea dx, repeat_msg

mov ah, 9

int 21h

mov ah, 1

int 21h

cmp al, 'Y'

je repeat_loop ; Repeat if 'Y' or 'y'

; Exit program

mov ah, 4Ch

int 21h

main endp

end main

.model small

.stack 100h

.data

buffer db 255 dup ('$') ; Buffer to store the input string

msg db 10, 13, "Enter a string (maximum 255 characters): $"

result db 10, 13, "Reversed string: $"

.code

main proc

mov ax, @data

mov ds, ax

lea dx, msg

mov ah, 9

int 21h ; Display the prompt message

mov si, 0 ; Initialize index for input string

read_loop:

mov ah, 1

int 21h ; Read a character from keyboard

cmp al, 13 ; Check if ENTER key pressed

je reverse_string ; If yes, jump to reverse the string

mov [buffer+si], al ; Store the character in the buffer

inc si

jmp read_loop ; Repeat the loop

reverse_string:

; Print the reversed string

lea dx, result

mov ah, 9

int 21h

; Pop characters from the buffer and print in reverse order

dec si

print_loop:

mov dl, [buffer+si]

mov ah, 2

int 21h

dec si

jge print_loop ; Repeat until all characters are printed

mov ah, 4Ch

int 21h ; Exit program

main endp

end main

.model small

.stack 100h

.data

buffer_size db 21       ; Buffer size (20 characters + RETURN)

input_buffer db 21 dup(0) ; Actual buffer for input

count_numbers db 0      ; Counter for integer numbers (initially 0)

.code

main proc

mov ax, @data

mov ds, ax

; Prompt the user for input

lea dx, prompt_message

mov ah, 09h

int 21h

; Read input from the user

lea dx, buffer_size

mov ah, 0Ah

int 21h

; Process the input buffer

lea si, input_buffer

mov cx, buffer_size

xor al, al  ; Clear AL (accumulator) for counting

read_loop:

lodsb       ; Load the next character into AL

cmp al, 13  ; Check if it's the ENTER key

je done     ; If ENTER, exit the loop

cmp al, '0' ; Check if it's a digit

jb not_digit

cmp al, '9'

ja not_digit

inc count_numbers ; Increment the count if it's a digit

not_digit:

loop read_loop

done:

; Display the count of integer numbers

lea dx, result_message

mov ah, 09h

int 21h

; Display the count (convert to ASCII)

mov al, count_numbers

add al, '0'

mov dl, al

mov ah, 02h

int 21h

; Exit

mov ah, 4Ch

int 21h

prompt_message db 10, 13, "Enter a sequence of characters (max 20): $"

result_message db 10, 13, "Number of integer digits: $"

endp

end main